HiFliQ S-type FPLC Columns

Datasheet

Protein Ark HiFliQ columns pre-packed and ready to use with sulphonic acid propyl linked (S) coupled Agarose resin for rapid Ion Exchange (IEX) purification of positively charged proteins under native conditions. Available in a range of HiFliQ column sizes with high ligand density and high binding capacity. Compatible with all common chromatography HPLC and FPLC instruments (including ÄKTA™ FPLC’s), and low pressure pumps and syringes using an appropriate adaptor.

| Resin Specification: |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item: | HiFliQ10-S-1 |  | HiFliQ20-S-1 |  | HiFliQ50-S-1 |
| Column Volume: | 10 ml |  | 20 ml |  | 50 ml |
| Resin: | Sulphonic acid propyl linked Agarose |  | Sulphonic acid propyl linked Agarose |  | Sulphonic acid propyl linked Agarose |
| Typical Binding Capacity: | 500-700 mg/ml |  | 1-1.4 g/ml |  | 2.5 -3.0g/ml |
| Base Matrix: | 6% cross-linked Agarose |  | 6% cross-linked Agarose |  | 6% cross-linked Agarose |
| Mean Bead Size: | 90 µm |  | 90 µm |  | 90 µm |
| Recommended flow rate: | 10 ml/min |  | 10-20 ml/min |  | 10-50 ml/min |
| Max. operating pressure: | 0.5 MPa (72 psi) |  | 0.5 MPa (72 psi) |  | 0.5 MPa (72 psi) |
| External Dimensions: | 16mm ID x 50mm BH |  | 16mm ID x 100mm BH |  | 26mm ID x 100mm BH |
| Column Construction: | Acrylic |  | Acrylic |  | Acrylic |
| Top and bottom filters: | Polyamide |  | Polyamide |  | Polyamide |
| Top and bottom plugs: | Polypropylene |  | Polypropylene |  | Polypropylene |
| Inlet Port: | 10-32 (1/16”) Female |  | 10-32 (1/16”) Female |  | 10-32 (1/16”) Female |
| Outlet Port: | 10-32 (1/16”) Female |  | 10-32 (1/16”) Female |  | 10-32 (1/16”) Female |
| Storage (2-8°C): | 20% ethanol + 0.2M Sodium Acetate |  | 20% ethanol + 0.2M Sodium Acetate |  | 20% ethanol + 0.2M Sodium Acetate |

| Ordering Information: |  |  |
| --- | --- | --- |
| Product | Units | Order Code |
| 10 ml HiFliQ S Type FPLC column (1 x 10 ml) | 1 x 10ml | HiFliQ10-S-1 |
| 20 ml HiFliQ S Type FPLC column (1 x 20 ml) | 1 x 20 ml | HiFliQ20-S-1 |
| 50 ml HiFliQ S Type FPLC columns (1 x 50 ml) | 1 x 50 ml | HiFliQ50-S-1 |
| *Custom sizes also available |  |  |

Instruction of Use:

Each packed column is sealed with a pressured syringe in the bottom end of the column.

It is then placed in sealed plastic bag.

1. Cut the plastic bag to take out the packed column with care.

2. Follow the flow direction to clamp the column to a vertical stand.

3. Get the chromatography system ready for connection.

4. Unscrew the stop plug. Some liquid may come out.

5. Connect the column top to the inlet of the chromatography system.

6. Gently unhook the springs from the shaft top of the bottom syringe using balanced

force.

7. Twist with push-up force to unscrew the male-thread adaptor from the column top.

Keep this storage syringe for later use.

8. Connect the bottom side of the column to the chromatography system.

9. Run at reduced flowrates to wash away the storage solution then run at normal

flowrate until the column is equilibrated.

DO NOT EXCEED 5 BAR

Storage after use:

For short term storage, seal both ends of the column with stop plugs if it isn’t being used.

For longer term storage, follow the instructions below to store the column.

1. Fill the column with suitable storage solution. Typically it requires to pass at least 2

CV storage solution.

2. Stop the pump.

3. Suck the storage solution to fully fill the storage syringe (including the male-thread

and the tubing part). Leave the syringe upside-down and push out the air bubble.

Adjust the liquid level inside the syringe to 3.5 ml to 4 ml (for 0.8 to 1.1 cm ID

columns) or 11 to 12 ml (for 2.6 cm ID columns).

4. Carefully screw the male thread part of the syringe system to the outlet of the

column. Finger tight is enough.

5. Dis-connect the column top from the chromatography system.

6. Screw a 1/16” stop plug to seal the column top.

7. Use balanced force to hook the springs back the top shaft of the syringe.

8. Check and make sure no leakage.

9. Place the column in a cold room.